# Crane Creek HEC-RAS Narrative

The Federal Emergency Management Agency (FEMA) provided HEC-2 data for Crane Creek near Red I-77 in Columbia, SC. The HEC-2 project included two data files called CRANER.DAT and CRANEF-R.DAT and two output files called CRANER.OUT and CRANEF-R.OUT. The CRANER.DAT and CRANER.OUT files are the input and output files for the flood plains run. The CRANEF-R.DAT and CRANEF-R.OUT files are the input and output files for the encroachments run. The HEC-2 models were produced in the NGVD 29 datum.

Duplicate Effective Model

The CRANER.DAT file was imported into a HEC-RAS model since this was the flood plains run. HEC-RAS version 4.1.0 was used throughout the duplicate effective, corrected effective, and revised models. The imported HEC-2 model would not compute due to errors in the conversion between HEC-2 and HECRAS. For the model to compute, the following changes were made:

- All elevations were converted from the NGVD 29 datum to the NAVD 88 datum using a conversion factor of -1.0.

- Removed all flow change locations upstream of section 675.

- Removed all flow change locations downstream of section 620.5.

- Removed all sections and bridges/culverts upstream of section 669 (Per FIS: 66,900 / AV).

- Removed all sections and bridges/culverts downstream of section 556 (Per FIS: 55,600 / AM).

- Applied know water surface elevations at section 556 (Per FIS: 55,600 / AM) from the HEC-2 model (Elevations applied were in NAVD 88 datum).

- Changed the distance between the upstream section and the face on bridge for the existing I-77 box culvert from 0’ to 0.1’ and the bridge width from 310’ to 309.8’ since HEC-RAS requires a positive distance from the end of the bridge to the next section downstream.

- Adjusted elevation of ground point 594 at river station 613 from 364 to 264.

- Changed all low chord elevations to 0 and removed pier data for culvert modeled as special bridge at station 605.5. Added HEC-2 culvert data as a culvert in HEC-RAS.

Corrected Effective Model

The duplicate effective geometry file was modified to incorporate the more detailed field survey in the vicinity of the existing Red River Road bridge. The following changes were made:

- Removed FEMA sections 6 through 9 and replaced them at new locations with updated field survey data.

- The discharges from the HEC-2 model did not match the discharges in the published FEMA flood study data. The steady flow file was modified to reflect the published FEMA discharges, with flow changes at sections 8 and 14.

- The top of road was revised to reflect the new survey data.

- Incorporated existing bridge rail into the top of road elevations based on the field investigation.

- Changed bridge width from 34.8’ to 26’.

- Changed the weir coefficient from 2.5 to 3.0.

- Changed minimum weir flow from 502.20’ to 500.91’ since this is the sag elevation in the roadway and overtopping occurs here.

- Modified the embankment sides slope from 0 to 2 both upstream and downstream.

- Changed the distance between the upstream section and the face of bridge from 0.1’ to 126’.

- Modified the contraction and expansion coefficients for sections 4 through 7 and 10 through 14 from 0.3 and 0.5 to 0.1 and 0.3.

- Added ineffective flow areas to the bridge. The contraction and expansion ratios were set at 1:1 and were determined using methods recommended by the USACOE. The effective flow areas on the left are set past the bridge since overtopping occurs on the roadway. The ineffective flow areas are controlled by the drive entrance near sta. 108+00 LT.

- Changed pier data from one pier at 5.1’ wide to three piers at 1.5’ wide each.

- The HEC-2 project modeled the existing bridge and an exposed downstream sewer line as one bridge. The bridge and sewer line were modeled separately in the corrected effective. The distance from the downstream face of the existing bridge to the sewer line is 7’.

Unrestricted Model

The corrected effective geometry file was modified to model the natural flow path in vicinity of the existing Red River Road bridge. The following changes were made:

- Removed existing Red River Road bridge data.

- Modified contraction and expansion coefficients for section 9 from 0.3 and 0.5 to 0.1 and 0.3.

Proposed Model

The existing Red River Road bridge at section 7.5 will be replaced with a longer bridge 50’ (centerline to centerline) upstream at section 7.7. The revised bridge will not change the 100-year discharge. Therefore, the steady flow file was not modified. The corrected effective geometry file was modified as follows to reflect the new bridge:

- The roadway and low chord elevations were modified to reflect the proposed three span 20’ flat slab bridge. The first span is 30 feet long, the second span is 40 feet long, and the third span is 30 feet long.

- Modified the roadway width and distance to the upstream section to reflect the wider bridge.

- Modified the pier locations so the piers remained outside the normal water surface and modified the size to reflect the proposed 18 inch square piers. The drag coefficient also changed from 1.2 to 2.0.

- Added 2.0:1 abutment fill slopes.

- Adjusted ineffective flow areas to reflect proposed bridge. The contraction and expansion ratios were set at 1:1 and were determined using methods recommended by the USACOE.