# Little Jackson Creek HEC-RAS Narrative

The Federal Emergency Management Agency (FEMA) provided HEC-2 data in .pdf form for Little Jackson Creek near I-77 in Columbia, SC. The HEC-2 project included input data for the existing conditions and output data for the 100 year flood plains run and encroachment run. The HEC-2 models were produced in the NGVD 29 datum.

Duplicate Effective Model

The input data was hard punched into a HEC-RAS model. HEC-RAS version 4.1.0 was used throughout the duplicate effective, corrected effective, and revised models. The input HEC-2 model would not compute due to errors in the conversion between HEC-2 and HECRAS. For the model to compute, the following changes were made:

- All elevations were converted from the NGVD 29 datum to the NAVD 88 datum using a conversion factor of -1.0.

- Removed all flow change locations upstream of section 138

- Removed all flow change locations downstream of section 63.

- Removed all sections and bridges/culverts upstream of section 138.

- Removed all sections and bridges/culverts downstream of section 63.

- Applied know water surface elevations at section 63 from the FEMA FIS profile (Elevations applied were in NAVD 88 datum).

- Changed the distance between the upstream section and the face of bridge for the existing Barbara Road bridge at section 106.9 from 0’ to 0.1’ and the bridge width from 10’ to 9.8’ since HEC-RAS requires a positive distance from the end of the bridge to the next section downstream.

- Changed the distance between the upstream section and the face of bridge for the existing southbound I-77 bridge at section 108.5 from 0’ to 0.1’ and the bridge width from 20’ to 19.8’ since HEC-RAS requires a positive distance from the end of the bridge to the next section downstream.

- Changed the distance between the upstream section and the face of bridge for the existing northbound I-77 bridge at section 109.7 from 0’ to 0.1’ and the bridge width from 20’ to 19.8’ since HEC-RAS requires a positive distance from the end of the bridge to the next section downstream.

- Changed the distance between the upstream section and the face of bridge for the existing Legrand Road box culvert at section 119.5 from 0’ to 0.1’ and the bridge width from 50’ to 49.8’ since HEC-RAS requires a positive distance from the end of the bridge to the next section downstream.

- Changed all low chord elevations to 0 and removed pier data for culvert modeled as special bridge at station 119.5. Added HEC-2 culvert data as a culvert in HEC-RAS.

- Deleted cross section data at river stations 106.8, 106.9, 108.3, 108.5, 109.5, and 109.7, to remove pier data that was modeled by the bottom of the river elevations in HEC-2. These widths and elevations were used to input pier data at each bridge section.

Corrected Effective Model

The duplicate effective geometry file was modified to incorporate the more detailed field survey in the vicinity of the existing Red River Road bridge. The following changes were made:

- Removed FEMA sections 6 through 9 and replaced them at new locations with updated field survey data.

- The discharges from the HEC-2 model did not match the discharges in the published FEMA flood study data. The steady flow file was modified to reflect the published FEMA discharges, with flow changes at sections 8 and 14.

- The top of road was revised to reflect the new survey data.

- Incorporated existing bridge rail into the top of road elevations based on the field investigation.

- Changed bridge width from 34.8’ to 26’.

- Changed the weir coefficient from 2.5 to 3.0.

- Changed minimum weir flow from 502.20’ to 500.91’ since this is the sag elevation in the roadway and overtopping occurs here.

- Modified the embankment sides slope from 0 to 2 both upstream and downstream.

- Changed the distance between the upstream section and the face of bridge from 0.1’ to 126’.

- Modified the contraction and expansion coefficients for sections 4 through 7 and 10 through 14 from 0.3 and 0.5 to 0.1 and 0.3.

- Added ineffective flow areas to the bridge. The contraction and expansion ratios were set at 1:1 and were determined using methods recommended by the USACOE. The effective flow areas on the left are set past the bridge since overtopping occurs on the roadway. The ineffective flow areas are controlled by the drive entrance near sta. 108+00 LT.

- Changed pier data from one pier at 5.1’ wide to three piers at 1.5’ wide each.

- The HEC-2 project modeled the existing bridge and an exposed downstream sewer line as one bridge. The bridge and sewer line were modeled separately in the corrected effective. The distance from the downstream face of the existing bridge to the sewer line is 7’.

Unrestricted Model

The corrected effective geometry file was modified to model the natural flow path in vicinity of the existing Red River Road bridge. The following changes were made:

- Removed existing Red River Road bridge data.

- Modified contraction and expansion coefficients for section 9 from 0.3 and 0.5 to 0.1 and 0.3.

Proposed Model

The existing Red River Road bridge at section 7.5 will be replaced with a longer bridge 50’ (centerline to centerline) upstream at section 7.7. The revised bridge will not change the 100-year discharge. Therefore, the steady flow file was not modified. The corrected effective geometry file was modified as follows to reflect the new bridge:

- The roadway and low chord elevations were modified to reflect the proposed three span 20’ flat slab bridge. The first span is 30 feet long, the second span is 40 feet long, and the third span is 30 feet long.

- Modified the roadway width and distance to the upstream section to reflect the wider bridge.

- Modified the pier locations so the piers remained outside the normal water surface and modified the size to reflect the proposed 18 inch square piers. The drag coefficient also changed from 1.2 to 2.0.

- Added 2.0:1 abutment fill slopes.

- Adjusted ineffective flow areas to reflect proposed bridge. The contraction and expansion ratios were set at 1:1 and were determined using methods recommended by the USACOE.