SOUTH CAROLINA DEPARTMENT OF TRANSPORTATION

Project Survey Report

SR-46-103  York  County

Thu Apr 14 10:15:03 2005

______________________________________________________________________________

Mainline SR-46-103 Oak Park Road                    50+00.00 - 76+00.00

______________________________________________________________________________

Tracking Information              Survey Options

File #:46.169B.01                 Units: English

Pin  #: 31919                     North Orientation: Grid

Item #: 0                         State Plane Coor: Yes

County: York                      Vertical Datum: NAVD-88

Data Files                        Survey Totals

Data File: 4631919.NEW            Alignments:       3

Report File: 4631919.DOC          Length:           0.751 MI

Station Equalities:

Reference Points: 0

Data Points:      1236

Survey Crew Members

James P. Hinson              A.E. III

Charles L. Johnson           A.E. II

Floyd A. Sweatt, Jr.         A.E. II

Daryl Strong                 E.T. III

Marcus L. Williamson         E.T. III

James S. Yarborough          E.T. II

Joe O. Ellis                 E.T. II

Durwood B. Frazer            E.T. II

Closure of Traverse  1 / 90,000

Survey Started: 3-28-05       Survey Ended: 4-14-05

Comments:

Add ? to Northing for State Plane Coordinates

Add ? to Easting for State Plane Coordinates

______________________________________________________________________________

Section Index

______________________________________________________________________________

Controls Points                 1 - 29

Topography Points               1000 - 1401

Break Lines Points              20000 - 20804

______________________________________________________________________________

Control Points

______________________________________________________________________________

1, 1117469.81968, 1966775.98530,  559.06573,CP 1  8 INCH NAIL

2, 1117687.47365, 1967326.71865,  543.84259,CP 2  8 INCH NAIL

3, 1117915.38227, 1967897.17240,  540.51123,CP 3  8 INCH NAIL

4, 1117906.82808, 1968420.07393,  536.37915,CP 4  8 INCH NAIL

5, 1117872.67987, 1969233.69403,  549.93701,CP 5  8 IN NAIL

______________________________________________________________________________

Summary of Horizontal Alignments

______________________________________________________________________________

Alg 01: MAINLINE SR-46-103              50+00.00 - 76+00.00   0.492 MI

6       (POT) POINT ON TANGENT               50+00.00

7       (POT) POINT ON TANGENT               60+00.13

8       (POT) POINT ON TANGENT               61+37.23

9       (PC)  POINT OF CURVE                 61+63.41

10       (POC) POINT ON CURVE                 61+95.19

11       (PI)  POINT OF INTERSECTION          63+04.02

12       (PT)  POINT OF TANGENCY              64+40.40

14       (POT) POINT ON TANGENT               66+91.62

15       (POT) POINT ON TANGENT               76+00.00

Alg 02:   Creek Traverse              20+00.00 - 30+58.51   0.200 MI

16       (POT) POINT ON TANGENT               20+00.00

17       (PI)  POINT OF INTERSECTION          20+73.27

18       (PI)  POINT OF INTERSECTION          22+14.76

19       (PI)  POINT OF INTERSECTION          23+59.63

20       (PI)  POINT OF INTERSECTION          25+07.63

21       (PI)  POINT OF INTERSECTION          25+71.77

22       (PI)  POINT OF INTERSECTION          26+49.24

23       (PI)  POINT OF INTERSECTION          29+24.02

24       (POT) POINT ON TANGENT               30+58.51

Alg 03:   Outfall              10+00.00 - 13+05.40   0.058 MI

25       (POT) POINT ON TANGENT               10+00.00

26       (POT) POINT ON TANGENT               13+05.40

______________________________________________________________________________

Details of Alignment   1  -  MAINLINE  SR-46-103

______________________________________________________________________________

______________________________________________________________________________

6       POT   Sta. = 50+00.0000  Bearing = N 68^00'15" E     1000.1268'

N = 1117448.11662  E = 1966770.74002

______________________________________________________________________________

7       POT   Sta. = 60+00.1268  Bearing = N 68^00'15" E     137.1055'

N = 1117822.70331  E = 1967698.06864

______________________________________________________________________________

8       POT   Sta. = 61+37.2323  Bearing = N 68^00'15" E     26.1811'

N = 1117874.05470  E = 1967825.19439

Equality:     Sta = 25+71.7733  Aln = #2  PN = 21     PI

______________________________________________________________________________

9       PC    Sta. = 61+63.4134  Bearing = N 68^00'15" E      SEE NEXT

N = 1117883.86055  E = 1967849.46980

______________________________________________________________________________

10      POC   Sta. = 61+95.1890

N = 1117895.04132  E = 1967879.21004

______________________________________________________________________________

11      PI    Sta. = 63+04.0163

N = 1117936.52186  E = 1967979.83839

Delta    =  24^16'09" Right    Chord    =  274.9227

Radius   =  653.9269           External =  14.9450

Degree   =  08^45'42"          Back     =  S 68^00'15" W

ArcLength=  276.9888           Ahead    =  S 87^43'36" E

Tangent  =  140.6029           ChordBear=  N 80^08'20" E

______________________________________________________________________________

12      PT    Sta. = 64+40.4022  Bearing = S 02^16'24" W     653.9270'

N = 1117930.94451  E = 1968120.33065

______________________________________________________________________________

13      RP

N = 1117277.53221  E = 1968094.39110

______________________________________________________________________________

14      POT   Sta. = 66+91.6207  Bearing = S 87^43'36" E     908.3793'

N = 1117920.97934  E = 1968371.35145

Equality:     Sta = 10+00.0000  Aln = #3  PN = 25     POT

______________________________________________________________________________

15      POT   Sta. = 76+00.0000

N = 1117884.94633  E = 1969279.01578

______________________________________________________________________________

______________________________________________________________________________

Details of Alignment   2  -  Creek Traverse

______________________________________________________________________________

______________________________________________________________________________

16      POT   Sta. = 20+00.0000  Bearing = S 54^28'32" E     73.2679'

N = 1118370.23205  E = 1967851.73902

______________________________________________________________________________

17      PI    Sta. = 20+73.2679  Bearing = S 07^59'49" E     141.4952'

N = 1118327.65965  E = 1967911.36937

______________________________________________________________________________

18      PI    Sta. = 22+14.7631  Bearing = S 42^02'42" W     144.8624'

N = 1118187.54038  E = 1967931.05389

______________________________________________________________________________

19      PI    Sta. = 23+59.6255  Bearing = S 11^23'22" W     148.0051'

N = 1118079.96263  E = 1967834.03777

______________________________________________________________________________

20      PI    Sta. = 25+07.6305  Bearing = S 18^31'47" E     64.1428'

N = 1117934.87222  E = 1967804.80991

______________________________________________________________________________

21      PI    Sta. = 25+71.7733  Bearing = S 18^31'47" E     77.4708'

N = 1117874.05470  E = 1967825.19439

Equality:     Sta = 61+37.2323  Aln = #1  PN = 8      POT

____________________________________________________________________________

22      PI    Sta. = 26+49.2441  Bearing = S 26^00'44" W     274.7809'

N = 1117800.60011  E = 1967849.81449

______________________________________________________________________________

23      PI    Sta. = 29+24.0250  Bearing = S 48^59'58" E     134.4854'

N = 1117553.65464  E = 1967729.30536

______________________________________________________________________________

24      POT   Sta. = 30+58.5104

N = 1117465.42308  E = 1967830.80178

______________________________________________________________________________

______________________________________________________________________________

Details of Alignment   3  -  Outfall

______________________________________________________________________________

______________________________________________________________________________

25      POT   Sta. = 10+00.0000  Bearing = N 19^37'49" E     305.4027'

N = 1117920.97934  E = 1968371.35145

Equality:     Sta = 66+91.6207  Aln = #1  PN = 14     POT

______________________________________________________________________________

26      POT   Sta. = 13+05.4027

N = 1118208.63210  E = 1968473.95118

______________________________________________________________________________