# SCDOT
PUBLIC DISPLAY CADD MANUAL

# STANDARDS AND PROCESSES FOR CREATING PUBLIC DISPLAYS USED IN MEETINGS

## Preface

To the User,

In accordance with the SCDOT Public Participation Plan1 and Federal regulations; The SCDOT conducts Public Information meetings (PIM) and Public Hearings (PH) throughout the state.  PIMs allow the public an opportunity to comment on proposed projects.  The first step in facilitating this process is effective communication and visual aids.  Visual aids used for PIMs are referred to as public meeting displays.  This manual is detailed for creating public meeting displays that are professional, effective, and efficient.

While creating a public meeting display, it is important to understand the language gap between SCDOT, Consulting Parties2 and its Stakeholders3.  Engineering plans and designs, by nature, contain more information than can be understood by the general public.  Saturation of information will lead to confusion; much of the information contained in these plans and designs will need to be removed in order to make the information easily understandable.  The most effective public meeting displays communicate the intended scope of a project with the least amount of information.  SCDOT has developed recommended layouts and details for public-friendly displays.

The SCDOT provides this manual as a guide for utilizing CADD files and programs to create public meeting displays.  Standards and procedures set forth in this manual have been developed through preference, and alternative methods exist.  Regardless of the procedure, the completed public meeting display should resemble the standards detailed at the end of this manual.  If you find this manual to be in error or would like to contribute to its development, please contact the SCDOT Design Automation team for assistance.

Sincerely, 
SCDOT Design Automation

*It is important to note that these displays are only intended for conceptual purposes only.

References: 
(1) A plan implemented and maintained by the SCDOT that provides direction for effective participation in the development of the SCDOT's transportation plans and programs.
(2) Local, state, regional, and federal agencies, transportation advocacy groups, environmental interest groups, and other entities that provide guidance and input into the planning process, identify critical transportation issues, and determine the goals and principles for implementation; 2016 SCDOT Public Participation Plan, pg. 2
(3) A person or group of people who affects or can be affected by transportation decisions or actions; 2016 SCDOT Public Participation Plan, pg. 2

## Table of Contents

- Lifecycle Outline

  - Preparation

  - Production

  - Presentation

- Preparation

  - Create Display Draft

    - What data should be included?

    - How should the data be communicated?

  - Obtain Necessary Files

    - Cadd support resources

    - Project specific files

  - Create Folder Structure

- Production

  - Consolidate Design Files

  - Eliminate Redundant Data

  - Re-map Levels

  - Attach Aerial Photography

  - Create Shapes

  - Customize Border

  - Customize Labels

  - Customize Legend

  - Combine Files for Display

- Presentation

  - SCDOT resources

  - Plotting

  - Microstation IPLOT

- Standards

  - Border & Dimensions

  - Legend

  - Labels

  - Do’s

  - Don’ts

  - Typical Sections

## lifecycle outline

The lifecycle of a public display can be divided into three stages; Preparation, Production and Presentation.  Each stage is broken down into several tasks.

The Preparation stage begins once a public display is requested.  This stage includes obtaining all of the necessary files and data for creating your display.  These files will include CADD drawings and aerial photography.  If you are unsure which CADD files you will need, please coordinate with the project and/or design managers.

After you obtain all of the necessary files, the Production stage may begin.  Production will include editing the copied design files and customizing your display.  There are several ways to accomplish this task and every display will be different.  This manual will explain some of the most typical methods used when preparing your CADD files.  Additional resources such as the SCDOT Roadway CADD Manual may be helpful.

Once you have finalized your public display, you are now ready to complete the assignment in the Presentation stage.  Some public displays will need to be plotted on paper while others will be shown digitally; either way, you will need pdf files for the presentation.

## preparation stage

Before you begin creating your public display, it is a good idea to consider the best way to present your project.  Some projects - typically intersection improvements - will only need one display.  Larger projects such as widening or relocation will require multiple displays.  You should spend some time determining what scale(s) to use and how to orient your border(s) in relation to the project area.

Coordinate obtaining the project CADD files through the project manager or road designer.  You will want proposed designs with references and aerial photography.  Depending on the project, you might also need to include additional data.  Alternative alignments or designs are good examples.

You will also need the PIM_DisplayTemplates.dgn file provided through the SCDOT Doing Business / CADD Design internet web link.  This file will contain several border sizes to choose from and a standard map legend.  If you maintain your own copies of this file, please ensure that your borders and legends match the display standards.

The SCDOT also maintains an internal library of typical sections for different roadways.  These images are for different lane configurations and may not be applicable for your project.  If you need to make modifications or new cross sections; Please contact SCDOT Design Automation for assistance.

The last step in the Preparation stage is setting up the folder structure.  In most cases you will have several files and they will need to be stored in an efficient manner.  Having a well-organized folder structure will provide the following benefits.

- Saving/referencing backup files (this will help if you make any mistakes, the CADD files become corrupt or you need to revert to previous versions of alternative designs)

- Organizing your files (help you keep track of your work & make it easier for someone else to find what they need)

- Name your files correctly (reduce extra work and simplify your work flow)

See the figure below for preferred folder structure. Also, please do not use spaces or special characters in folder names.

## production stage

One of the first things you need to do is collect all of the drawing data into one file.  The best way to do this is to create a new-blank drawing and reference any drawings you will need. Then configure your level display to show only levels/data that you want on your display.  A list of commonly required elements/data is provided in the Standards Section of this document.  After your Level Display is set, select all of your references in the Reference Manager and then "Merge into Master."  This will consolidate all of the pertinent data into one file.

After merging all of your references into one file, unnecessary data will most likely still exist.  This data will include text, lines and shapes that are not ideal for a public display.  Take your time and delete the unnecessary data.  Again, this is why it is a good idea to back up your work.  This process can be simplified by fully utilizing the "Element Selection Tool" and "Level Display.”

After the file has been appropriately cleaned up, in the next step, we will re-map the levels over to the predefined Public Involvement Meeting Levels.  Each level has been Pre-defined with a standard color and priority for your convenience.  This process can seem tedious in nature if done manually.  The SCDOT Design Automation team maintains a generic level re-mapping file for PIM displays.  Because every project is different, this file probably won't be set up for all of your levels but should convert most of them - automating most of your work.  This file can be customized for your continued use, so long as you adhere to the standard color scheme set in this manual.  Please let the SCDOT Design Automation team know of any changes that could be made to our generic file.

Aerial photography will also need to be included.  The SCDOT maintains a server dedicated for Aerial photography.  If you are not familiar with this resource please refer to the SCDOT Roadway CADD Manual.

Similar to aerial photography, shape elements can greatly increase the visual perception of your display.  Shape elements should be given a filled transparency so that aerial photography is still visible.  Here is a common list of elements that should be overlaid with a shape element.

- New Pavement

- Existing Pavement (circumstantial)

- New Rights of Way

- Existing Rights of Way

- Easements

- Raised Medians

- New Bridges & Structures

- Existing Bridges & Structures (circumstantial)

Additionally, you will need to check your pavement markings for accuracy.  The level "PIM_PAVE_MARK" is set to color "30" for white markings.  You will need to manually set any yellow markings to the color "17."  Please use the SCDOT Standard Drawings as a reference.

With most of the editing being complete now, it is time to begin creating your display file.  This task will be accomplished by referencing the necessary files into a new-blank file.  Since you want to modify your reference files as little as possible, you might find it easiest to copy the standard legend directly into your display file.

The aerial photography and design file should line up with each other; while, the border and labels will need to be placed manually.  This will include translating, rotating and possibly scaling the border and legend.  Labels should be placed using a standard configuration and nomenclature.  Consideration should also be given for the size and spacing of each label - Similar labels should have matching symbology.  Several examples of proper labeling can be found in the Standards Section of this manual.

If you only have one display, it might be easier to place the labels directly into your design.dgn, rather than in your display.dgn.  This will let you move the border around without moving the labels.  If you need multiple display.pdf's you can create multiple border and label files to simplify your work.  At this point the standard workflow should be similar to this.

- Create design.dgn file (merge to master, remap levels, create shapes) done previously.

- Create display.dgn file and reference design.dgn, border and legend.

- Move, align and size your border to the desired location.

- Set display to show desired elements.

- Place labels on applicable elements.

- Check legend is representative of display.

- Use Iplot to export .pdf file.

- Check file and adjust display.dgn as necessary.

- Use Iplot to export .pdf file for final use.

Repeat as necessary to create your public display(s).

## Presentation stage

SCDOT employees will present these displays to the public and will need dependable resources.  Displays should be created first, in pdf format.  The next several sections of this manual will detail how to create a pdf file.  The SCDOT Plotting Manual provides more detail and can be found on the Preconstruction Support web page.

Pdf files can be quickly generated using the standard print tools in Microstation.  Once you are satisfied with your display in pdf format, send a link to the file to Planstorage@scdot.org for printing.  Due to technical language and physical properties of the plotters, your display may appear slightly different between the printed version and the digital version.

When creating a pdf file of your display, do not change the dimensions.  For example, if your display border is 36" by 60" from edge to edge; create a pdf file that is 36" x 60" edge to edge.  Maintaining the same dimensions will help the visual integrity.

Here are the general steps for creating a pdf of your display using Microstation.

- Verify the dimensions of your display.

- Place a fence over the RD_SHT_IplotShp (dotted line around border sheet).

- From the Main Menu navigate to File > IPLOT

- Select the “browse” option for the printer.

- Set the Printer to "\\SMPLTSERVC-08-1\D8PLC-TDS700_505_IPLOT"
If you have not mapped to this plotting server, you can find instructions through the SCDOT Preconstruction Support web page.

- Check the paper size, orientation and scale settings.  Make sure the Size X & Y values will match the dimensions of what you want to print.

- Open the “Select Plotting Files” from the File menu option or pressing (Ctrl + F)

- Choose the color table named “SCDOT_color.tbl” file from the following location:

\\nts\hq\caddstandards\SCDOT-Bentley\Site\Iplot\Pen_Tables

- Once you are ready, select "Export PDF…"

- Allow Microstation to process the file.  Aerial photography will require a majority of your computer's RAM so be patient.

- When the Export PDF window opens up, select the following settings and click "Create PDF"

- Set the location and name of your pdf and click "Save"

- The file will open in your default pdf viewer application and can be checked.

## Standards

The SCDOT provides several set border sheets for use in creating your public meeting displays.  You might find that there is not one particular border that best fits your project.  If you need to change the dimensions of your border, please do not change the x-y relationship of the elements contained in your border.  For example; the SCDOT logo should not be skewed.  If you are preparing a paper or canvas display then you will need to consider the dimensions of your plotter.  In all cases, utilize the standard border style for your public displays.  The idea behind these standard colors and line styles is that the display depicts what an actual road looks like so that it is easily relatable to the public.  These are meant to be a guide as all projects are different and therefore colors may need to be adjusted to accommodate the details of your project.

Fig. 1.1 Standard SCDOT public display border

The legend provided in Figure 1.2 contains most of the common features that should be displayed.  It is possible that you will need to add or remove some of the legend when creating your display.  Always make sure your display legend is well organized and shown correctly when plotted.  Do not include items in your display that are not shown in the legend and vice versa.  Design elements that fall into any of the categories from figure 1.3 need to be included in your display.  These colors were developed so that those with color blindness could still differentiate the lines.

Fig 1.2 Standard SCDOT public display map legend

Fig 1.3 SCDOT Public Display Microstation Levels and Descriptions

*Represents a layer that has to be on the display if the layer exists within the project

Every display should have labels on significant landmarks (i.e. roads, buildings, parks, neighborhoods etc.)  These labels should be placed relative to each other so that the display is clear and concise.  Similar features in you display should have similar labels.  You should also utilize a standard rotation scheme for linear features.  The pattern labels provided with the generic border and legend file are sized in relation to the border sheet.  In situations where there is not enough room to place your labels, consider reducing their size.

Fig. 1.4 Proper rotation of labels for linear features

Be sure to complete the information on the left-hand side of the border sheet.  Project Location needs to  denote the county in which the project is located.

Also, the Map Legend needs to be complete and accurate to match all of the contents in your display.

Contact information for the project manager should also be included.

The project name and road names, along with the road number should be shown across the top of your border.

Also, ensure the date in the bottom right-hand corner is correct.  This date should be the date of the meeting.

## do’s

## don’ts

## Typical Section Images

Typical sections should also be included with your display.  The SCDOT maintains a library with several examples that are ready for use.  If you cannot find a typical section that matches your design, please feel free to contact the SCDOT Design Automation team for assistance.  Two examples for your reference are shown below.