Synchro v7

OPTIONS

Map setting

Intersection paths on

Network Settings

Peak Hour Factor – 0.90

Yellow – 4 sec

Red – 2 sec

Offset style – Begin of Yellow

Min split through – 22 sec

Min split left – 15 sec

Simulation taper length – 100 ft

Crosswalk width – 10 ft

Uncheck lead/lag optimize

Actuated uncord

Detector Templates

Left – Det1 50, size 30

Thru – Det1 255, Det2 385

Generally for side streets, use loops at stop bar

(leading to 0)

Edit template phases – 2  SB, 4 WB

Simtraffic

Intervals and volumes

Seed 10 min*, record 60 or record 4 15-min intervals

* for large networks seeding should cover saturating network

Place file named defaults.syn in the trafficware directory where Synchro is installed