| Upland 9 (SM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # DATA FORM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ROUTINE WETLAND DETERMINATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1987 COE Wetlands Delineation Manual) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Project/Site: |  |  |  |  |  |  |  |  | SC 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 3/09/05 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Applicant/Owner: |  |  |  |  |  |  |  |  | SCDOT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | County: |  | Berkeley |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Investigator: |  |  |  |  |  |  |  |  | EcoScience Corporation / Davis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | State: |  | SC |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Do Normal Circumstances exist on the site? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  | Community ID: |  |  |  | Disturbed/Maintained |  |  |  |  |  |  |  |  |
|  |  |  | Is the site significantly disturbed (Atypical Situation)? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  | Transect ID: |  |  |  | SM04 |  |  |  |  |  |  |  |  |
|  |  |  | Is the area a potential Problem Area? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  | Plot ID: |  |  |  | upland |  |  |  |  |  |  |  |  |
|  |  |  | (If needed, explain on reverse.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VEGETATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dominant Plant Species |  |  |  |  |  |  |  |  |  |  |  |  | Stratum |  |  |  | Indicator |  |  |  |  |  | Dominant Plant Species |  |  |  |  |  |  |  |  |  | Stratum |  |  | Indicator |  |  |  |  |  |
|  |  |  | 1. | Morella cerifera |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  | FAC+ |  |  |  |  |  | 9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. | Smilax rotundifolia |  |  |  |  |  |  |  |  |  |  |  | V |  |  |  | FAC |  |  |  |  |  | 10. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. | Sabal minor |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  | FACW |  |  |  |  |  | 11. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 4. | Rubus sp. |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  | - |  |  |  |  |  | 12. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 6. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 7. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Percent of Dominant Species that are OBL, FACW or FAC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (excluding FAC-). 75-100% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ### HYDROLOGY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Recorded Data (Describe in Remarks): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wetland Hydrology Indicators: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Stream, Lake, or Tide Gauge |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Primary Indicators: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Aerial Photographs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inundated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Saturated in Upper 12 Inches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | No Recorded Data Available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Water Marks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Drift Lines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sediment Deposits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Field Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Drainage Patterns in Wetlands |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Secondary Indicators (2 or more required): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Depth of Surface Water: |  |  |  |  |  |  |  |  |  | - |  |  |  | (in.) |  |  |  |  |  |  |  |  | Oxidized Root Channels in Upper 12 Inches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Water-Stained Leaves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Depth to Free Water in Pit: |  |  |  |  |  |  |  |  |  | 12+ |  |  |  | (in.) |  |  |  |  |  |  |  |  | Local Soil Survey Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FAC-Neutral Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Depth to Saturated Soil: |  |  |  |  |  |  |  |  |  | 12+ |  |  |  | (in.) |  |  |  |  |  |  |  |  | Other (Explain in Remarks) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Upland 9 (SM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### SOILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Map Unit Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (Series and Phase): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Drainage Class: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Field Observations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Taxonomy (Subgroup): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confirm Mapped Type? |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |
|  |  | Profile Descriptions: Depth |  |  |  |  |  |  |  |  |  |  |  |  | Matrix Color |  |  |  |  |  |  |  | Mottle Colors |  |  |  |  |  |  | Mottle Abundance/ |  |  |  |  | Texture, Concretions, |  |  |  |  |  |  |  |  |  |
|  |  | (inches) |  |  |  |  |  |  |  |  | Horizon |  |  |  | (Munsell Moist) |  |  |  |  |  |  |  | (Munsell Moist) |  |  |  |  |  |  | Size/Contrast |  |  |  |  | Structure, etc, |  |  |  |  |  |  |  |  |  |
|  |  | 0-3 |  |  |  |  |  |  |  |  |  |  |  |  | 10yr 3/1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sandy loam |  |  |  |  |  |  |  |  |  |
|  |  | 3-9 |  |  |  |  |  |  |  |  |  |  |  |  | 10yr 3/2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sandy loam |  |  |  |  |  |  |  |  |  |
|  |  | 9-12 |  |  |  |  |  |  |  |  |  |  |  |  | 2.5y 3/2 |  |  |  |  |  |  |  | 2.5y 6/6 |  |  |  |  |  |  | 30% |  |  |  |  | sandy loam |  |  |  |  |  |  |  |  |  |
|  |  | Hydric Soil Indicators: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Histosol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Concretions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Histic Epipedon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | High Organic Content in Surface Layer in Sandy Soils |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Sulfidic Odor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Organic Streaking in Sandy Soils |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Aquic Moisture Regime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Listed on Local Hydric Soils List |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Reducing Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Listed on National Hydric Soils List |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Gleyed or Low-Chroma Colors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other (Explain in Remarks) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### WETLAND DETERMINATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Hydrophytic Vegetation Present? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | No |  |  | (Check) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (Check) |  |  |  |  |  |  |  |
|  |  | Wetland Hydrology Present? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Hydric Soils Present? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | No |  |  |  |  |  |  |  | Is this Sampling Point Within a Wetland? |  |  |  |  |  |  |  |  |  | Yes |  | No |  |  |  |  |  |
|  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approved by HQUSACE 3/92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forms version 1/02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |