Wetland 31 (TN)

DATA FORM

ROUTINE WETLAND DETERMINATION

(1987 COE Wetlands Delineation Manual)

| Project/Site: | SC 41 – Wando Bridge |  |  | Date: | 3/15/05 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Applicant/Owner: | SCDOT |  |  | County: | Berkeley/Charleston |  |
| Investigator: | M. Thomas - EcoScience |  |  | State: | SC |  |
| Do Normal Circumstances Exist on the Site? |  | Yes | No | Community ID: |  | PSS1/PFO4 |
| Is the site significantly disturbed (Atypical)? |  | Yes | No | Transect ID: |  | TN07 |
| Is the area a potential problem area? |  | Yes | No | Plot ID: |  | Wetland |

VEGETATION

| Dominant Plant Species |  |  | Stratum |  | Indicator |  | Dominant Plant Species |  |  | Stratum |  | Indicator |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Acer rubrum |  | C |  | FAC |  | 9. |  |  |  |  |  |  |
| 2. | Liquidambar styraciflua |  | C |  | FAC+ |  | 10. |  |  |  |  |  |  |
| 3. | Pinus taeda |  | C |  | FAC |  | 11. |  |  |  |  |  |  |
| 4. | Carex (Spp.) |  | H |  |  |  | 12. |  |  |  |  |  |  |
| 5. | Simalix rotundifolia |  | V |  | FAC |  | 13. |  |  |  |  |  |  |
| 6. | Sabal palmetto |  | T |  | FAC |  | 14. |  |  |  |  |  |  |
| 7. | Erianthus giganteus |  | H |  | FACW |  | 15. |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  | 16. |  |  |  |  |  |  |
| Percent of Dominant Species that are OBL, FACW or FAC (excluding FAC-) 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |

HYDROLOGY

|  |  |  |  |  | Primary Wetland Hydrology Indicators: |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Recorded Data (Describe in Remarks) |  |  |  | Inundated |
|  |  | Stream, Lake or Tide Gauge |  |  | x | Saturated in Upper 12 Inches |
|  |  | Aerial Photographs |  |  |  | Water Marks |
|  |  | Other |  |  |  | Drift Lines |
|  | x | No Recorded Data Available |  |  |  | Sediment Deposits |
|  |  |  |  |  |  | Drainage Patterns in Wetlands |
|  |  |  |  |  | Secondary Indicators: (2 or more required): |  |
| Field Observations: |  |  |  |  |  | Oxidized Root Channels in Upper 12 Inches |
| Depth of Surface Water: |  |  |  | (in.) |  | Water-Stained Leaves |
| Depth to Free Water in Pit: |  |  | 8 | (in.) |  | Local Soil Survey Data |
| Depth to Saturated Soil: |  |  | 7 | (in.) | x | FAC-Neutral Test |
|  |  |  |  |  |  | Other (Explain in Remarks) |
| Remarks: |  |  |  |  |  |  |

Wetland 31 (TN)

SOILS

| Map Unit Name (Series and Phase): |  |  |  |  |  |  | Meggett Loam |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Taxonomy (Subgroup): |  |  |  |  | Typic Albaqualfs |  |  |  |  |  |  |  |  |  |  |
| Drainage Class: |  |  |  |  | PD |  |  |  |  |  |  |  |  |  |  |
| Field Observations Confirm Mapped Type: Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Profile Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Depth Matrix Color Mottle Colors Mottle Texture, Concretions (inches) Horizon (Munsell Moist) (Munsell Moist) Abundance/Contrast Structure, etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 - 2 |  |  | A |  | 2.5Y 3/1 |  |  |  |  |  |  |  | Fine, sandy loam |  |
|  | 2 – 12+ |  |  | B |  | 10YR 6/1 |  |  | 2.5Y 6/6 |  |  | 10% |  | Course sand |  |
| Hydric Soil Indicators: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Histosol |  |  |  |  |  |  |  |  | Concretions |  |  |  |  |
|  |  | Histic Epipedon |  |  |  |  |  |  |  |  | High Organic Content in Surface layer in Sandy Soils |  |  |  |  |
|  |  | Sulfidic Odor |  |  |  |  |  |  |  |  | Organic Streaking in Sandy Soils |  |  |  |  |
|  | x | Aquic Moisture Regime |  |  |  |  |  |  |  |  | Listed on Local Hydric Soils List |  |  |  |  |
|  |  | Reducing Conditions |  |  |  |  |  |  |  | x | Listed on National Hydric Soils List |  |  |  |  |
|  | x | Gleyed or Low-Chroma Colors |  |  |  |  |  |  |  |  | Other (Explain in Remarks) |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

WETLAND DETERMINATION

| Hydrophytic Vegetation Present? Yes No |  |
| --- | --- |
| Wetland Hydrology Present? Yes No | Is this Sampling Point Within a Wetland? |
| Hydric Soils Present? Yes No | Yes No |
| Remarks: Medium quality freshwater wetland |  |