The USACE 404 Individual Permit conditions will be updated upon permit approval.

| Permit & No. | Permit Conditions | SCDOT Responsibility | Contractor Responsibility |
| --- | --- | --- | --- |
| BCM-1 | The permittee is required to keep adjacent critical areas free of construction materials, litter, debris, chemical contaminants, etc. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-2 | Any disturbed critical area adjacent to the construction site must be restored to original contours and conditions upon completion. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-3 | Only clean, earthen materials free of all potential sources of pollution may be used as fill in the aquatic impact area. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-4 | The project must be in compliance with any applicable floodplain, stormwater, land disturbance, shoreline management guidance or riparian buffer ordinances. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-5 | An as-built survey of the structure(s) must be submitted to the Department [SCDES BCM] within 90 days from completion of construction. The survey must be performed by a registered land surveyor, must show all components of the structure(s), and must list the starting and ending coordinates of the structure(s) in the SC State Plane Coordinate System, which can be obtained by survey-grade Global Positioning System equipment. |  | The CONTRACTOR shall comply with this commitment. This as-built shall be for the structures in/or adjacent to areas where critical area impacts are depicted and authorized. |
| BCM-6 | The applicant must implement BMPs that will minimize erosion and migration of sediments on and off the project site during and after construction. These practices should include the use of appropriate grading and sloping techniques, mulches, hay bales, silt fences, or other devices capable of preventing erosion, migration of sediments, and bank failure. All disturbed land surfaces and sloped areas affected by the project must be stabilized. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-7 | All efforts must be made to protect existing native riparian vegetation in and along shoreline areas. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-8 | Where plantings are prescribed, the applicant shall use native plantings appropriate for the ecoregion and should exclude plant species found on the exotic pest plant council list: https://seeppc.org/southcarolina/SCEPPC_LIST2014finalOct.pdf. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-9 | Once the project is initiated, it must be carried to completion in an expeditious manner, to the greatest extent practicable, to minimize the period of disturbance to the environment. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-10 | Mitigation for 0.33 acres of impacts to marsh/tidal critical areas, will be provided through the purchase of 4.46 mitigation credits from an approved mitigation bank. The proof of purchase of the 4.46 tidal wetland mitigation credits must be submitted to the Department prior to beginning work on the project. | The SCDOT will comply with this commitment. | The CONTRACTOR shall be responsible for completing any necessary permit modifications based on their final design. The CONTRACTOR will also be responsible for securing any additional credits from a USACE approved Mitigation Bank. The CONTRACTOR shall submit draft permit modifications to the SCDOT Environmental Services Office (ESO) for coordination with USACE and BCM. The CONTRACTOR shall make any necessary edits to the permit modification as needed by the SCDOT, USACE, or BCM. The CONTRACTOR shall submit proof of purchase of mitigation credits as required. |
| BCM-11 | In the event that any historic or cultural resources and/or archaeological materials are found during the course of work, the applicant must notify the State Historic Preservation Office and the South Carolina Institute of Archaeology and Anthropology. Historic or cultural resources consist of those sites listed in the National Register of Historic Places and those sites that are eligible for the National Register. Archaeological materials consist of any items, fifty years old or older, which were made or used by man. These items include, but are not limited to, stone projectile points (arrowheads), ceramic sherds, bricks, worked wood, bone and stone, metal and glass objects, and human skeletal materials. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-12 | The applicant has agreed to a monitoring plan during construction for cultural resources identified as being adjacent to the project. See additional coordination letter on September 23, 2025 from SCDAH, SHPO Project Number 22-EJ0143, SCDOT Project No. P04131. |  | The CONTRACTOR shall comply with this commitment. The CONTRACTOR shall ensure appropriate staff are present to comply with the monitoring requirements. |
| BCM-13 | The applicant has agreed to the boundaries of 38CH0319 and a 25' buffer must be marked on all construction documents and noted as "Historic Cemetery" and reference SC Code of Laws 16-17-600. The applicant should avoid any ground disturbance or temporary use of the cemetery and buffer area for staging equipment or materials {Per SCDAH, SHPO Project Number 22-EJ0143, SCOOT Project No. P041314). |  | The CONTRACTOR shall comply with this commitment. |
| BCM-14 | The project must be consistent with State Stormwater Permitting requirements during and post construction for protection of water quality. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-15 | All construction BMPs must be installed, inspected and maintained to hold sediment onsite and to protect any adjacent or downstream critical area, wetlands and waters through the life of the project. Upon completion of construction activities, all disturbed (including undeveloped) areas, including those impacted for access, must be immediately stabilized. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-16 | The applicant agrees to coordinate with SCDNR if state protected species are encountered during construction. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-17 | The applicant agrees to implement the following Best Management Practices be incorporated into the design of all bridges: a. All necessary measures will be taken to prevent oil, tar, trash, debris, and other pollutants from entering the adjacent waterway and/or wetland b. Bridge decks will be designed to keep runoff velocities low and control pollutant loadings c. The use of scuppers on bridges will be minimized and all deck drainage will be conveyed to land for treatment in detention ponds and infiltration systems to mitigate pollution loads d. Conveyance systems will be designed to withstand the velocities of projected peak discharge |  | The CONTRACTOR shall comply with this commitment. |
| BCM-18 | The applicant agrees to limit the removal of vegetation to what is necessary for the construction of the proposed improvements. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-19 | Prior to land disturbance and impacts to jurisdictional wetlands of 4.9 acres {3.53 acres of freshwater wetlands, and 1.37 acres of protected freshwater wetlands {Under SAC-26-1992- 762)), applicant must fully comply with all general, special and regional terms and conditions of the US Army Corps of Engineers Permit {SAC-2022-01082 and SAC-26-1992-762) as well as SCDES requirements for Water Quality and Coastal Zone Certifications. | SCDOT will comply with this commitment. | The CONTRACTOR shall comply with this commitment. |
| BCM-20 | Mitigation for the impacts to 4.9 acres of wetlands will be provided through the purchase of 38.83 freshwater wetland credits and an additional 230 freshwater wetland credits for impacts to protected wetlands under previous Permit SAC-26-1992-762. The proof of purchase of the 38.83 freshwater wetland credits and 230 freshwater wetland credits {to compensate for the removal of protective covenants on 9.553 acres of freshwater wetlands) must be submitted to the Department prior to beginning work on the project. | The SCDOT will comply with this commitment. |  |
| BCM-21 | If the activity authorized herein is not constructed or completed within five years of the date of issuance, this permit shall automatically expire. A request, in writing, for an extension of time shall be made not less than thirty days prior to the expiration date. | SCDOT will comply with this commitment. | The CONTRACTOR shall comply with this commitment. The CONTRACTOR shall notify prepare the necessary extension documentation and notify the ESO 120 days prior to the permit expiration date of the need for a permit extension. The CONTRACTOR shall coordinate with the ESO to obtain the permit extension. |
| BCM-22 | All authorized work shall be conducted in a manner that minimizes any adverse impact on fish, wildlife and water quality. |  | The CONTRACTOR shall comply with this commitment. |
| BCM-23 | The permit construction placard or a copy of the placard shall be posted in a conspicuous place at the project site during the entire period of work. |  | The CONTRACTOR shall comply with this commitment. The CONTRACTOR shall coordinate with the SCDOT ESO to request the placard. |
|  | INTENTIONALLY BLANK |  |  |